40 Pin Connector:

| Pin # | Name | Function |
| --- | --- | --- |
| 9 | CDH1_3V3 | CDH 1 3.3 Voltage |
| 11 | CDH2_3V3 | CDH 2 3.3 Voltage |
| 36 | GND | System Ground |
| 38 | CANL | CAN Low |
| 40 | CANH | CAN High |

SWD:

| Pin # | MCU Pin # | Name | Function |
| --- | --- | --- | --- |
| 1 | N/A | GND | System Ground |
| 2 | N/A | VREF | Direct Connection to 3.3v Reference (Bypasses Safety) |
| 3 | 49 | SWCLK | System Clock |
| 4 | 46 | SWDIO | Data In/Out |
| 5 | 7 | NRST | Reset. Pull low to reset chip |

MCU Pinout:

| Pin # | MCU # | Name | Function | Secondary Function |
| --- | --- | --- | --- | --- |
| 14 | PA0 | MCU#_FRAM3_F | FRAM 3 Fault (IN ¦ Active HIGH) | FRAM 3 Reset (OUT ¦ Active LOW) |
| 15 | PA1 | MCU#_FRAM2_F | FRAM 2 Fault (IN ¦ Active HIGH) | FRAM 2 Reset (OUT ¦ Active LOW) |
| 16 | PA2 | USART2_# | UART Connection between boards |  |
| 17 | PA3 | USART2_# | UART Connection between boards |  |
| 20 | PA4 | MCU#_FRAM1_F | FRAM 1 Fault (IN ¦ Active HIGH) | FRAM 1 Reset (OUT ¦ Active LOW) |
| 21 | PA5 | MCU_MR | Mux Reset (Active ¦ High) |  |
| 22 | PA6 | MCU_SEL | MCU 2 if HIGH |  |
| 23 | PA7 | NC |  |  |
| 41 | PA8 | CAN_F | CAN Fault (IN ¦ Active HIGH) | MCU# CAN Reset (OUT ¦ Active LOW) |
| 42 | PA9 | S | CAN Silent Mode (OUT ¦ Active HIGH) |  |
| 43 | PA10 | SHDN | CAN Shutdown Mode (OUT ¦ Active HIGH) |  |
| 44 | PA11 | RXD | CAN Receive (Data IN) |  |
| 45 | PA12 | TXD | CAN Transmit (Data OUT) |  |
| 46 | PA13 | SWDIO | Programing pin for SWD |  |
| 49 | PA14 | SWCLK | Programing Pin SWD Clock |  |
| 50 | PA15 | NC |  |  |
| 26 | PB0 | NC |  |  |
| 27 | PB1 | NC |  |  |
| 28 | PB2 | MCU#_WD | Watchdog Pulse (OUT) |  |
| 55 | PB3 | MCU#_B0 | Boot zero recovery pin |  |
| 56 | PB4 | MCU#_CS3 | FRAM 3 SPI Chip Select (OUT ¦ Active HIGH) |  |
| 57 | PB5 | MCU#_WP3 | FRAM 3 Write Protection (OUT) |  |
| 58 | PB6 | MCU#_I2C1_SCL | I2C1 SCL |  |
| 59 | PB7 | MCU#_I2C1_SDA | I2C1 SDA |  |
| 61 | PB8 | MCU#_CS2 | FRAM 2 SPI Chip Select (OUT ¦ Active HIGH) |  |
| 62 | PB9 | MCU#_WP2 | FRAM 2 Write Protection (OUT) |  |
| 29 | PB10 | MCU#_CS1 | FRAM 1 SPI Chip Select (OUT ¦ Active HIGH) |  |
| 33 | PB12 | MCU#_WP1 | FRAM 1 Write Protection (OUT) |  |
| 34 | PB13 | MCU#_SPI2_SCK | SPI2 SCK |  |
| 35 | PB14 | MCU#_SPI2_MISO | SPI2 MISO |  |
| 36 | PB15 | MCU#_SPI2_MOSI | SPI2 MOSI |  |
| 2 | PC13 | LED D3 | Diag LED 3 |  |
| 3 | PC14 | LED D2 | Diag LED 2 |  |
| 4 | PC15 | LED D1 | Diag LED 1 |  |
| 5 | PH0 | LCK1_2 | AND_Lock Key 1 (OUT ¦ Active HIGH) |  |
| 6 | PH1 | LCK2_IN | Reset opposite MCU (OUT ¦ Active HIGH Pulse) |  |
| 60 | PH3 | LCK1_OUT | BOOT 0 Input Pin |  |
| 54 | PD2 | LCK1_1 | AND_Lock Key 2 (OUT ¦ Active HIGH) |  |